-----------------------------------------------------------------

The following is a news release from Master Bond Inc. You have received it because you are listed as an editor for your publication.

Attached to this email is a low resolution version of the photograph that is included in the press kit for this product.

High resolution version of this image and files with the body text of this release in Word, HTML and text formats are available at https://www.masterbond.com/newsrelease/ep3htsda-1.

-----------------------------------------------------------------

## FOR IMMEDIATE RELEASE

Electrically Conductive Die Attach Adhesive Has Very High Thermal Conductivity

Master Bond EP3HTSDA-1 is a single part, no mix epoxy adhesive designed primarily for die attach applications. It exhibits a die shear strength of 20-22 kg-f and has a high thermal conductivity of over 40-45 BTU•in/ft2•hr•°F [5.7-6.5 W/(m•K)]. “This 100% solids formulation has an ideal viscosity and flow for die attach, will not ‘tail’ and can be easily dispensed automatically” says Rohit Ramnath, senior product engineer. “Additionally it has an unlimited working life at room temperature and will cure in 20-30 minutes at 250°F or 5-10 minutes at 300°F.”

Master Bond EP3HTSDA-1 is NASA low outgassing approved. Dimensionally stable, it has a Shore D hardness of 75-85 and is engineered to withstand thermal cycling and shock. This epoxy system adheres well to metals, ceramics, and silicon dies. Service operating temperature range is from -80°F to +400°F.

This product contains silver fillers and has a volume resistivity of <0.001 ohm-cm. It retains conductivity upon aging and provides excellent moisture and chemical resistance. This compound is formulated for applications in the communication, aerospace, medical, electronic, automotive, and defense industries. Master Bond EP3HTSDA-1 is available for use in syringe applicators.

Master Bond Die Attach Epoxy Adhesives

Master Bond EP3HTSDA-1 is a single component NASA low outgassing approved die attach adhesive featuring high thermal conductivity and low volume resistivity. Read more about Master Bond’s die attach epoxy adhesives at https://www.masterbond.com/industries/die-attach-epoxy-adhesives or contact Tech Support. Phone: +1-201-343-8983 Fax: +1-201-343-2132 Email: technical@masterbond.com.

Note to Editors:

For a full product description, please visit: http://www.masterbond.com/tds/ep30htsda-1

CONTACT

James Brenner, Marketing Manager

Email: jbrenner@masterbond.com

Tel: +1-201-343-8983

Fax: +1-201-343-2132

MASTER BOND INC.

154 Hobart Street

Hackensack, NJ 07601-3922

Web: www.masterbond.com

# # #