----------------------------------

The following is a news release from Master Bond Inc.  You have received it because you are listed as an editor for your publication.

Attached to this email is a low resolution version of the photograph that is included in the press kit for this product.

High resolution versions of this image and files with the body text of this release are available at http://www.masterbond.com/mbnr.html?nr=ep30lte-lo.

----------------------------------

FOR IMMEDIATE RELEASE

Low Thermal Expansion Epoxy 
Passes NASA Low Outgassing Tests

HACKENSACK, NJ – April 19, 2011 – Formulated with a special blend of polymeric and inorganic materials, Master Bond EP30LTE-LO features unparalleled dimensional stability and sets new standards of performance for bonding, sealing, casting and coating applications in the aerospace, electrical, electronic, chemical, optical, and computer industries. This two component epoxy combines outstanding performance with a very low thermal expansion coefficient, a significant breakthrough from the high thermal expansion that is typical of most epoxy compounds.

Serviceable over the wide temperature range of 4K to +250°F [4K to +121°C], EP30LTE-LO cures at room temperature within 48-72 hours. Faster cures are possible at elevated temperatures. This 100% reactive epoxy features an exceptionally low coefficient of thermal expansion of 15-18 x 10-6 in/in/°C and a low shrinkage rate of less than 0.0002 in/in. It bonds well to metals, glass, ceramics, wood, vulcanized rubbers and most plastics, and produces bonds with a tensile strength of 6,000-7,000 psi.

This electrically insulative system resists thermal cycling and chemicals including water, fuels, and many acids, bases, salts and organic solvents. It has a long working life of 45-75 minutes for a 100 gram mass.

EP30LTE-LO has a 10 to 1 mix ratio by weight and can be easily applied with a spatula, knife, trowel, brush or paint roller. It can also be readily cast as thick as 1 inch without undue exotherm developing. EP30LTE-LO is available in 1/2 pint, pint, quart, gallon and 5 gallon container kits.

Master Bond Low Outgassing Epoxy Systems

Master Bond's low outgassing epoxy, EP30LTE-LO, features a low thermal expansion coefficient with solid electrical insulation properties. Read more about Master Bond's low outgassing epoxies at http://www.masterbond.com/lp/tabs/tp_pp_lowoutgas.html or contact Technical Support by phone at +1-201-343-8983, by fax at +1-201-343-2132 or by e-mail at technical@masterbond.com.

CONTACT:

James Brenner, Marketing Manager

Email: jbrenner@masterbond.com

Tel: 201-343-8983

Fax: 201-343-2132

MASTER BOND INC.

154 Hobart Street

Hackensack, NJ  07601-3922

Web: www.masterbond.com

# # #