This email was sent to (%email%)

Unsubscribe at: http://www.masterbond.com/phplist/?p=unsubscribe&id=2

Subscribe at: http://www.masterbond.com/phplist/?p=subscribe&id=2

----------------------------------

The following is a news release from Master Bond Inc.  You have received it because you are listed as an editor for your publication.

Attached to this email is a low resolution version of the photograph that is included in the press kit for this product.

High resolution versions of this image and files with the body text of this release are available at http://www.masterbond.com/mbnr.html?nr= uv10tklo-2.

----------------------------------

FOR IMMEDIATE RELEASE

UV Curable Adhesive/Sealant Meets

NASA Low Outgassing Requirements

HACKENSACK, NJ – Mar 18, 2010 – Master Bond Inc., Hackensack, NJ, is pleased to announce the development of a unique one component UV curable modified epoxy system called UV10TKLO-2.  This formulation offers optical clarity, high physical strength properties, and has outstanding chemical resistance.  It is not oxygen inhibited and cures rapidly at room temperature when exposed to UV light.  Maximum UV absorption takes place in the 320-365nm range.

Master Bond UV10TKLO-2 is easy to apply and has an advantageous high viscosity.  It has passed the rigorous tests to meet low outgassing specifications as per NASA ASTM E-595.  It can be used in a wide array of optical, electro-optical and vacuum applications that until now have been off limits for UV curable compounds.  To optimize low outgassing properties, UV10TKLO-2 should be preheated to 50°C for 15 minutes prior to use and then allowed to cool to room temperature.

Particularly noteworthy is its superior electrical insulation properties.  It has a volume resistivity of >1014 ohm cm and a dielectric constant of 3.6 at 60Hz.  UV10TKLO-2 has a service temperature range of -60°F to 300°F.  It is available for use in half pint, pint, quart, and gallon containers.  Shelf life in original unopened containers is 6 months at 75°F.

CONTACT:

James Brenner, Marketing Manager

Email: jbrenner@masterbond.com

Tel: 201-343-8983

Fax: 201-343-2132

MASTER BOND INC.

154 Hobart Street

Hackensack, NJ  07601-3922

Web: www.masterbond.com

# # #